Kabelboom conversie Mazda 323 BG GTX → Mazda 323F BG

| Mazda 323F BG |  | Mazda 323 BG GTX |
| --- | --- | --- |
| Stekker X03 |  |  |
| BLACK | GROUND | MASSA – GROUND |
| BROWN/YELLOW | CLUTCH SWITCH | X04 - BROWN/YELLOW |
| BLUE/YELLOW | POWERSTEERING PRES. | X04 - BLUE/YELLOW |
| WHITE/GREEN | STOPLIGHT SWITCH | X04 - WHITE/GREEN |
| BLACK/BLUE | REARWINDOW DEFROST | X04 – BLACK/BLUE |
| Stekker X04 |  |  |
| BLACK/ORANGE | WATERTHERMO. SENSOR | X04 – BLACK/ORANGE |
| GREEN/BLACK | AC AMP. | X03 – GREEN/ORANGE |
| RED/BLACK | COMBINATION SWITCH | X03 – RED/BLACK |
| BLACK/PINK | FUELPOMP RELAIS | GREEN/RED ( fat cable ) |
| BLUE/RED | GENERALPOWER | X04 – BLUE/RED |
| BLUE/BLACK | AC RELAIS | X03 – BLUE/BLUE |
| BLACK/GREEN | COOLING FAN RELAIS | X04 – BLACK/GREEN |
| ORANGE/BLUE | BLOWER SWITCH | X04 – ORANGE/BLUE |
| YELLOW/BLUE | TACHOMETER | X04 – YELLOW/BLUE |

MARKER : When done correct you can cut away the left over plugs and finish up.  Do not let any blank copper open or against each other.

Made by 
Adri Hoogenbosch
H-P-C.