121 LX/Suntop, 1.3,, 1323 cc, 1988-91, Engine Code B3,

Ignition data

121 LX/Suntop, 1.3
Ignition system type: El-i
Ignition coil make/type: Bo/0 221 122 012
Supply voltage: 12 - Volts without ballast resistor
Primary resistance: 0.8-1.3 Ohms
Secondary resistance: 6000-30000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 2   1/850 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 2   1/850 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR5ES
Electrode gap: 0.75-0.85 mm

Fuel System

121 LX/Suntop, 1.3
Fuel type: L/U
Fuel pump type/pressure: Mech/0.28-0.35 bar
Engine idle speed: 850   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5 Vol %

Engine Data

121 LX/Suntop, 1.3
Valve clearance - inlet: 0.30 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 10-14 bar
Oil pressure: 3.0-4.0/3000 bar/eng/rpm
Radiator pressure cap: 0.75-1.05 bar/type
Drive belt 1 - size: 9.5x888 KDB888 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

121 LX/Suntop, 1.3
Alternator output: 45/2500 Amps/eng/rpm
Regulated output: 14.4-15.0 Volts
Battery: 12/33 V/Ah
Starter motor make/type: Mit/0.85 kW

Other data

121 LX/Suntop, 1.3
Clutch operation type: Cable
Clutch adjustment: 1.5-2.5/Lever mm
Cambelt failure: Damage
Cambelt Part Number: B63212205
Tyre pressures: 2.0(28)f 2.0(28)r bar (psi)