323 (BF), 1.3,, 1323 cc, 1987-89, , Engine Code B3

Ignition data

323 (BF), 1.3
Ignition system type: El-i
Ignition coil make/type: n / a
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 1.0-1.3 Ohms
Secondary resistance: 6000-30000 Ohms
Distributor make/type: Hit/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 2   1/850 [AT=2   1/1000] Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 2   1/850 [AT=2   1/1000] Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR5ES
Electrode gap: 0.75-0.85 mm

Fuel System

323 (BF), 1.3
Fuel type: L/U
Fuel pump type/pressure: Mech/0.27-0.34 bar
Engine idle speed: 850   50 [AT=1000   50] rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5-2.5 Vol %

Engine Data

323 (BF), 1.3
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 9.5-13.6 bar
Oil pressure: 3.4-4.4/3000 bar/eng/rpm
Radiator pressure cap: 0.75-1.05 bar/type
Drive belt 1 - size: 9.5x960 KDB960 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

323 (BF), 1.3
Alternator output: 60/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/35 V/Ah
Starter motor make/type: Mit/0.85 kW

Other data

323 (BF), 1.3
Clutch operation type: Cable
Clutch adjustment: 9-15/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: B63212205
Tyre pressures: 2.0(28)f 1.8(26)r bar (psi)