323 (BF), 1500 ,, 1490 cc, 1985-87, , Engine Code E5

Ignition data

323 (BF), 1500
Ignition system type: El-i
Ignition coil make/type: n / a
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 1.0-1.3 Ohms
Secondary resistance: 6000-30000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 4   1/850 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 4   1/850 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: Champion
Spark plug type: RN9YC
Electrode gap: 0.8 mm

Fuel System

323 (BF), 1500
Fuel type: L/U
Fuel pump type/pressure: Mech/0.20-0.31 bar
Engine idle speed: 850   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5-2.0 Vol %

Engine Data

323 (BF), 1500
Valve clearance - inlet: 0.25 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 12 bar
Oil pressure: 3.5-4.5/3000 bar/eng/rpm
Radiator pressure cap: 0.75-1.05 bar/type
Drive belt 1 - size: 9.5x960 KDB960 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

323 (BF), 1500
Alternator output: 55-60/3000 Amps/eng/rpm
Regulated output: 14.4-15 Volts
Battery: 12/35 V/Ah
Starter motor make/type: Lu/LRS415

Other data

323 (BF), 1500
Clutch operation type: Cable
Clutch adjustment: 232/P.Travel mm
Cambelt failure: Damage
Cambelt Part Number: n / a
Tyre pressures: 2.0(28)f 1.8(26)r bar (psi)