323 (BG), 1.3 LX 8V ,, 1323 cc, 1989-, Engine Code B3E ,

Ignition data

323 (BG), 1.3 LX 8V
Ignition system type: El-i
Ignition coil make/type: Han/-
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 0.77-0.95 Ohms
Secondary resistance: 10000-16000 Ohms
Distributor make/type: n / a
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 0   1/850 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR6ES11
Electrode gap: 1.0-1.1 mm

Fuel System

323 (BG), 1.3 LX 8V
Fuel type: U Cat
Fuel pump type/pressure: Elec/4.5-6.0 bar
Engine idle speed: 850   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Fuel System

323 (BG), 1.3 LX 8V
Fuel type: U Cat
Fuel pump type/pressure: Elec/4.5-6.0 bar
Engine idle speed: 850   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

323 (BG), 1.3 LX 8V
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 9.7-13.9 bar
Oil pressure: 3-4/3000 bar/eng/rpm
Radiator pressure cap: 0.75-1.05 bar/type
Drive belt 1 - size: 9.5x888 KDB888 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

323 (BG), 1.3 LX 8V
Alternator output: 65/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/45 V/Ah
Starter motor make/type: Mit/12V/0.85 kW

Other data

323 (BG), 1.3 LX 8V
Clutch operation type: Hyd
Clutch adjustment: 0.7-3.3/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: BP02122050 [BP0112205B]
Tyre pressures: 2.1(30)f 1.9(27)r bar (psi)