323 (BG), 1.6i 16V ,, 1597 cc, 1991-, Engine Code B6,

Ignition data

323 (BG), 1.6i 16V
Ignition system type: El-ecu
Ignition coil make/type: Han/
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 0.81-0.99 Ohms
Secondary resistance: 10000-16000 Ohms
Distributor make/type: n / a
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 10   1/750 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BKR5E-11
Electrode gap: 1.0-1.1 mm

Fuel System

323 (BG), 1.6i 16V
Fuel type: U Cat
Fuel pump type/pressure: Mech/0.28-0.35 bar
Engine idle speed: 750   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5-2.5 Vol %

Engine Data

323 (BG), 1.6i 16V
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 7.8-11.1 bar
Oil pressure: 3.0-4.0/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.03 bar/type
Drive belt 1 - size: 9.5x900 KDB900 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

323 (BG), 1.6i 16V
Alternator output: 65/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Bo/0 001 108 072

Other data

323 (BG), 1.6i 16V
Clutch operation type: Hyd
Clutch adjustment: 0.3-3.3/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: BP0212205 [BP0112205B]
Tyre pressures: 2.0(28)f 1.8(26)r bar (psi)