323 Estate, 1500,, 1490 cc, 1982-87, , Engine Code D5

Ignition data

323 Estate, 1500
Ignition system type: CB
Ignition coil make/type: Lu/DLJ100
Supply voltage: 3 + Volts with ballast resistor
Primary resistance: 3.1 Ohms
Secondary resistance: 10000-30000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: 0.5 mm
Dwell angle: 49-55 Deg
Ignition Timing - unleaded: 8/750 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 8/750 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: Pulley
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR5ES
Electrode gap: 0.8 mm

Fuel System

323 Estate, 1500
Fuel type: L/U
Fuel pump type/pressure: Mech/0.2-0.27 bar
Engine idle speed: 750   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5   0.5 Vol %

Engine Data

323 Estate, 1500
Valve clearance - inlet: 0.25 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 8.4-12 bar
Oil pressure: 3.5-4.5/3000 bar/eng/rpm
Radiator pressure cap: 0.9 bar/type
Drive belt 1 - size: 9.5x960 KDB960 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

323 Estate, 1500
Alternator output: 50/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/33 V/Ah
Starter motor make/type: Lu/LRS415

Other data

323 Estate, 1500
Clutch operation type: Cable
Clutch adjustment: 230/P.Travel mm
Cambelt failure: Damage
Cambelt Part Number: n / a
Tyre pressures: 1.7(24)f 1.7(24)r bar (psi)