323 Turbo, 4x4 ,, 1597 cc, 1986-87, Engine Code B6DOHC,

Ignition data

323 Turbo, 4x4
Ignition system type: El-i
Ignition coil make/type: n / a
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 1.0-1.3 Ohms
Secondary resistance: 6000-30000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: n / a Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 12   1/800 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BCPR6E
Electrode gap: 0.8 mm

Fuel System

323 Turbo, 4x4
Fuel type: L
Fuel pump type/pressure: Elec/2.45-2.85 bar
Engine idle speed: 800-900 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5   0.5 Vol %

Engine Data

323 Turbo, 4x4
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 7.6-10.8 bar
Oil pressure: 3-4/3000 bar/eng/rpm
Radiator pressure cap: 0.75-1.05 bar/type
Drive belt 1 - size: 8.5x888 KDB888 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

323 Turbo, 4x4
Alternator output: 60/3000 Amps/eng/rpm
Regulated output: 14.4-15.0 Volts
Battery: 12/45 V/Ah
Starter motor make/type: Mit/0.85 kW

Other data

323 Turbo, 4x4
Clutch operation type: Hyd
Clutch adjustment: 0.6-3.0/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: B66012205
Tyre pressures: 2.0(28)f 1.8(26)r bar (psi)