626 GLX, 2.0 12V ,, 1998 cc, 1987-92, ,Engine Code FE

Ignition data

626 GLX, 2.0 12V
Ignition system type: El-i
Ignition coil make/type: Han/-
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 1.04-1.27 Ohms
Secondary resistance: 7100-9700 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 6   1/800 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 6   1/800 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BCPR6E
Electrode gap: 0.7-0.8 mm

Fuel System

626 GLX, 2.0 12V
Fuel type: L/U +Cat
Fuel pump type/pressure: Mech/0.2-0.29 bar
Engine idle speed: 800-850 [AT=900-950] rpm
Fast idle speed: n / a rpm
CO level at idle speed: 2.0   0.5 Vol %

Engine Data

626 GLX, 2.0 12V
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 10.0-14.2 bar
Oil pressure: 2.9-3.9/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.03 bar/type
Drive belt 1 - size: PV/870 KIB875 mm/type
Drive belt 2 - size: PV/1010 KIB1015 mm/type

Electrical System

626 GLX, 2.0 12V
Alternator output: 70/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Mit/1.4 kW

Other data

626 GLX, 2.0 12V
Clutch operation type: Hyd
Clutch adjustment: 5-13/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: FE1M12205
Tyre pressures: 2.0(28)f 1.8(26)r bar (psi)