626, 1600 ,, 1587 cc, 1983-87, ,Engine Code F6

Ignition data

626, 1600
Ignition system type: El-i
Ignition coil make/type: Lu/DLJ100
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 1.03-1.27 Ohms
Secondary resistance: 10000-30000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 6   1/900 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 6   1/900 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR6ES
Electrode gap: 0.8 mm

Fuel System

626, 1600
Fuel type: L/U
Fuel pump type/pressure: Mech/0.2-0.25 bar
Engine idle speed: 825   25 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 2.0   1.0 Vol %

Engine Data

626, 1600
Valve clearance - inlet: 0.30 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 8.1-11.5 bar
Oil pressure: 3.4-4.5/3000 bar/eng/rpm
Radiator pressure cap: 0.9 bar/type
Drive belt 1 - size: 9.5x975 KDB975 mm/type
Drive belt 2 - size: PV/975 KJB975 mm/type

Electrical System

626, 1600
Alternator output: 30/2500 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Lu/LRS557

Other data

626, 1600
Clutch operation type: Cable
Clutch adjustment: 216/P.Travel mm
Cambelt failure: Damage
Cambelt Part Number: F80812205
Tyre pressures: 1.8(26)f 2.0(28)r bar (psi)