626, 2.0i ,, 1998 cc, 1986-87, , Engine Code FE-EGI

Ignition data

626, 2.0i
Ignition system type: El-i
Ignition coil make/type: Han/-
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 1.03-1.27 Ohms
Secondary resistance: 10000-30000 Ohms
Distributor make/type: Den/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: n / a Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 6   1/850 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR6ES
Electrode gap: 0.75-0.85 mm

Fuel System

626, 2.0i
Fuel type: L
Fuel pump type/pressure: Elec/2.4-2.8 bar
Engine idle speed: 850-900 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 2.0   0.5 Vol %

Engine Data

626, 2.0i
Valve clearance - inlet: 0.30 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 9.95-14.2 bar
Oil pressure: 2.94-3.92 bar/eng/rpm
Radiator pressure cap: 0.74-1.03 bar/type
Drive belt 1 - size: PV/875 KJB875 mm/type
Drive belt 2 - size: PV/1000 KJB100 mm/type

Electrical System

626, 2.0i
Alternator output: 53/2500 Amps/eng/rpm
Regulated output: 14.3-15.0 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Den/0.95 kW

Other data

626, 2.0i
Clutch operation type: Cable
Clutch adjustment: 11-17/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: FE0812205
Tyre pressures: 1.8(26)f 2.0(28)r bar (psi)