626, MX-6 ,, 1991 cc, 1992-, Engine Code FS

Ignition data

626, MX-6
Ignition system type: El-ecu
Ignition coil make/type: Han/-
Supply voltage: n / a
Primary resistance: 0.68-0.83 Ohms
Secondary resistance: 9600-14400 Ohms
Distributor make/type: Mit/
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 12   1/700 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BKR5E-11
Electrode gap: 1.0-1.1 mm

Fuel System

626, MX-6
Fuel type: U Cat
Fuel pump type/pressure: Elec/4.4-6.3 bar
Engine idle speed: 700   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

626, MX-6
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 8.2-11.8 bar
Oil pressure: 3.9-4.9/3000 bar/eng/rpm
Radiator pressure cap: 0.93-1.23 bar/type
Drive belt 1 - size: n / a mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

626, MX-6
Alternator output: 80/2750 Amps/eng/rpm
Regulated output: 14.4-14.7 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Mit/1.4 kW

Other data

626, MX-6
Clutch operation type: Cable
Clutch adjustment: 5-14/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: FS0112205A
Tyre pressures: 2.2(32)f 1.8(26)r bar (psi)