626, MX-6 ,, 2497 cc, ,1992- , Engine Code KL

Ignition data

626, MX-6
Ignition system type: El-ecu
Ignition coil make/type: Han/-
Supply voltage: n / a
Primary resistance: 0.58-0.86 Ohms
Secondary resistance: 11500-18500 Ohms
Distributor make/type: Mit/
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 10   1/650 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/R
Firing Order: 1-2-3-4-5-6
Spark plug make: NGK
Spark plug type: ZFR5F-11
Electrode gap: 1.0-1.1 mm

Fuel System

626, MX-6
Fuel type: U Cat
Fuel pump type/pressure: Elec/5.0-6.3 bar
Engine idle speed: 650   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

626, MX-6
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 9.8-14.0 bar
Oil pressure: 3.4-4.9/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.02 bar/type
Drive belt 1 - size: n / a mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

626, MX-6
Alternator output: 90/2750 Amps/eng/rpm
Regulated output: 14.4-14.7 Volts
Battery: 12/65 V/Ah
Starter motor make/type: Bo/0 001 108 026

Other data

626, MX-6
Clutch operation type: Cable
Clutch adjustment: 5-14/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: KL0112205
Tyre pressures: 2.2(32)f 1.8(26)r bar (psi)