929 2.0, ,, 1970 cc, 1978-81, Engine Code 2000/MA,

Ignition data

929 2.0,
Ignition system type: CB
Ignition coil make/type: Lu/DLJ100
Supply voltage: n / a
Primary resistance: 1.5 Ohms
Secondary resistance: 9000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: 0.5 mm
Dwell angle: 49-55 Deg
Ignition Timing - unleaded: n / a Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 8/700 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: Pulley
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR6ES
Electrode gap: 0.8 mm

Fuel System

929 2.0,
Fuel type: L
Fuel pump type/pressure: Elec/0.20-0.25 bar
Engine idle speed: 725   25 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 2.5-3.5 Vol %

Engine Data

929 2.0,
Valve clearance - inlet: 0.30 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 12 bar
Oil pressure: 3.5-4.5/3000 bar/eng/rpm
Radiator pressure cap: 0.9 bar/type
Drive belt 1 - size: 12.5x975 KEB975 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

929 2.0,
Alternator output: 40/2500 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/40 V/Ah
Starter motor make/type: Lu/LRS903

Other data

929 2.0,
Clutch operation type: Hyd
Clutch adjustment: 220/P.Travel mm
Cambelt failure: Damage
Cambelt Part Number: n / a
Tyre pressures: 1.8(26)f 1.8(26)r bar (psi)