E1600/B1600, Van/Pick-up ,, 1586 cc, 1982-, Engine Code B1600/NA,

Ignition data

E1600/B1600, Van/Pick-up
Ignition system type: CB
Ignition coil make/type: Lu/DLJ100
Supply voltage: 3.9 + Volts with ballast resistor
Primary resistance: 1.5 Ohms
Secondary resistance: 9000 Ohms
Distributor make/type: n / a
Contact breaker gap/air: 0.5 mm
Dwell angle: 49-55 Deg
Ignition Timing - unleaded: 8   1/800 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 8   1/800 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: Pulley
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR5ES
Electrode gap: 0.8 mm

Fuel System

E1600/B1600, Van/Pick-up
Fuel type: L/U
Fuel pump type/pressure: Mech/0.2-0.75 bar
Engine idle speed: 800   50/725   25 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 2.0   0.5 Vol %

Engine Data

E1600/B1600, Van/Pick-up
Valve clearance - inlet: 0.30 c mm (cold)
Valve clearance - exhaust: 0.30 c mm (cold)
Compression pressure: 8.9-11.9/8.3 bar
Oil pressure: 3.5-4.5/3000 bar/eng/rpm
Radiator pressure cap: 0.9 bar/type
Drive belt 1 - size: 12.5x925 KEB925 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

E1600/B1600, Van/Pick-up
Alternator output: n / a Amps/eng/rpm
Regulated output: 14-15.5 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Lu/LRS903

Other data

E1600/B1600, Van/Pick-up
Clutch operation type: Hyd
Clutch adjustment: S/Adj. mm
Cambelt failure: Damage
Cambelt Part Number: n / a
Tyre pressures: 3.0(43)f 2.6(37)r [B1600=1.8(26)f 4.3(61)r] bar (psi)