E2000, Van ,, 1998 cc, 1985-, Engine Code FE,

Ignition data

E2000, Van
Ignition system type: CB
Ignition coil make/type: Lu/DLJ100
Supply voltage: 3.0 + Volts with ballast resistor
Primary resistance: 3.1 Ohms
Secondary resistance: 6000-13000 Ohms
Distributor make/type: Mit/-
Contact breaker gap/air: 0.5 mm
Dwell angle: 49-55 Deg
Ignition Timing - unleaded: 6   1/800 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 6   1/800 Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: Pulley
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BPR6ES
Electrode gap: 0.8   0.05 mm

Fuel System

E2000, Van
Fuel type: L/U
Fuel pump type/pressure: Mech/0.2-0.3 bar
Engine idle speed: 825   25 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 1.5-2.5 Vol %

Engine Data

E2000, Van
Valve clearance - inlet: 0.30 h mm (hot)
Valve clearance - exhaust: 0.30 h mm (hot)
Compression pressure: 11.5 bar
Oil pressure: 3-4/3000 bar/eng/rpm
Radiator pressure cap: 0.75-1.05 bar/type
Drive belt 1 - size: 12.5x950 KEB950 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

E2000, Van
Alternator output: n / a Amps/eng/rpm
Regulated output: 14.4-15 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Mit/12v-0.85kW

Other data

E2000, Van
Clutch operation type: Hyd
Clutch adjustment: S/Adj. mm
Cambelt failure: Damage
Cambelt Part Number: FE1H12205
Tyre pressures: 2.8(39)f 2.8(39)r bar (psi)