MX-3, ,, 1598 cc, 1991-, Engine Code B6,

Ignition data

MX-3,
Ignition system type: El-ecu
Ignition coil make/type: Den/-
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 0.81-0.99 Ohms
Secondary resistance: 10000-16000 Ohms
Distributor make/type: Den/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 10   1/750 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/F
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BKR5E-11
Electrode gap: 1.0-1.1 mm

Fuel System

MX-3,
Fuel type: U Cat
Fuel pump type/pressure: Elec/2.7-3.1 bar
Engine idle speed: 750   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

MX-3,
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 9.8-12.8 bar
Oil pressure: 2.9-3.9/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.03 bar/type
Drive belt 1 - size: 9.5x900 KDB800 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

MX-3,
Alternator output: 70/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/60 V/Ah
Starter motor make/type: Bo/0 001 108 002

Other data

MX-3,
Clutch operation type: Hyd
Clutch adjustment: 5-15/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: BP0212205 [BP0112205B]
Tyre pressures: 2.0(28)f 2.0(28)r bar (psi)