MX-3, V6 ,, 1845 cc, 1991-, Engine Code K8,

Ignition data

MX-3, V6
Ignition system type: El-ecu
Ignition coil make/type: Den/-
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 0.58-0.86 Ohms
Secondary resistance: 11500-18500 Ohms
Distributor make/type: Den/-
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 10   1/670 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/R
Firing Order: 1-2-3-4-5-6
Spark plug make: NGK
Spark plug type: BKR5E-11
Electrode gap: 1.0-1.1 mm

Fuel System

MX-3, V6
Fuel type: U Cat
Fuel pump type/pressure: Elec/2.7-3.1 bar
Engine idle speed: 670   30 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

MX-3, V6
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 9.8-13.3 bar
Oil pressure: 3.3-4.9/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.03 bar/type
Drive belt 1 - size: n / a mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

MX-3, V6
Alternator output: 90/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/65 V/Ah
Starter motor make/type: Bo/0 001 114 015

Other data

MX-3, V6
Clutch operation type: Hyd
Clutch adjustment: 5-15/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: KL0112205
Tyre pressures: 2.0(28)f 2.0(28)r bar (psi)