MX-5, ,, 1597 cc, 1989- Engine Code B6 DOHC ,

Ignition data

MX-5,
Ignition system type: DIS
Ignition coil make/type: n / a
Supply voltage: 11 - Volts without ballast resistor
Primary resistance: 0.78-0.94 Ohms
Secondary resistance: 11200-15200 Ohms
Distributor make/type: n / a
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 10   1/850 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: Pulley
Firing Order: 1-3-4-2
Spark plug make: NGK
Spark plug type: BKR5E-11
Electrode gap: 1.0-1.1 mm

Fuel System

MX-5,
Fuel type: U Cat
Fuel pump type/pressure: Elec/2.65-3.14 bar
Engine idle speed: 850   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

MX-5,
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 9.3-13.2 bar
Oil pressure: 2.9-3.9/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.03 bar/type
Drive belt 1 - size: 12.5x900 KEB900 mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

MX-5,
Alternator output: 60/3000 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/32 V/Ah
Starter motor make/type: Mit/0.95 kW

Other data

MX-5,
Clutch operation type: Hyd
Clutch adjustment: 5-13/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: B65712205A
Tyre pressures: 1.8(26)f 1.8(26)r bar (psi)