RX7, ,,1573 cc ,, 1979-84, Engine Code 12A,

Ignition data

RX7,
Ignition system type: CB
Ignition coil make/type: Mit/-
Supply voltage: n / a
Primary resistance: 1.4L/1.5T Ohms
Secondary resistance: n / a Ohms
Distributor make/type: Mit/L/T
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: n / a Deg eng/rpm (without vacuum)
Ignition Timing - leaded: 5/750 Leading [20/750 Trailing] Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: Pulley
Firing Order: n / a
Spark plug make: NGK
Spark plug type: BR7ET
Electrode gap: 1.0 mm

Fuel System

RX7,
Fuel type: L
Fuel pump type/pressure: Elec/0.26-0.33 bar
Engine idle speed: 750   25 rpm
Fast idle speed: n / a rpm
CO level at idle speed: n / a Vol %

Engine Data

RX7,
Valve clearance - inlet: n / a
Valve clearance - exhaust: n / a
Compression pressure: 6 bar
Oil pressure: 4.5-5.5/- bar/eng/rpm
Radiator pressure cap: 0.9 bar/type
Drive belt 1 - size: 9.5x800 KDB800 mm/type
Drive belt 2 - size: 9.5x888 KDB888 mm/type

Electrical System

RX7,
Alternator output: 39/2500 Amps/eng/rpm
Regulated output: 14.1-14.7 Volts
Battery: 12/55 V/Ah
Starter motor make/type: Lu/LRS501

Other data

RX7,
Clutch operation type: Hyd
Clutch adjustment: 190/P.Travel mm
Cambelt failure: n / a
Cambelt Part Number: n / a
Tyre pressures: 1.9(27)f 1.9(27)r bar (psi)