Xedos 6, 2.0i ,, 1995 cc, 1992-, Engine Code KF,

Ignition data

Xedos 6, 2.0i
Ignition system type: El-ecu
Ignition coil make/type: Den/-
Supply voltage: 12 - Volts without ballast resistor
Primary resistance: 0.81-0.99 Ohms
Secondary resistance: 10000-15000 Ohms
Distributor make/type: n / a
Contact breaker gap/air: n / a mm
Dwell angle: n / a Deg
Ignition Timing - unleaded: 10   1 Deg eng/rpm (without vacuum)
Ignition Timing - leaded: n / a Deg eng/rpm (without vacuum)
Timing mark location: Pulley
No 1 plug location: O/S/R
Firing Order: 1-2-3-4-5-6
Spark plug make: NGK
Spark plug type: BKR6E-11
Electrode gap: 1.0-1.1 mm

Fuel System

Xedos 6, 2.0i
Fuel type: U Cat
Fuel pump type/pressure: Elec/5.0-6.4 bar
Engine idle speed: 670   50 rpm
Fast idle speed: n / a rpm
CO level at idle speed: 0.5 Max Vol %

Engine Data

Xedos 6, 2.0i
Valve clearance - inlet: Hyd
Valve clearance - exhaust: Hyd
Compression pressure: 14.7 bar
Oil pressure: 3.4/3000 bar/eng/rpm
Radiator pressure cap: 0.74-1.02 bar/type
Drive belt 1 - size: n / a mm/type
Drive belt 2 - size: n / a mm/type

Electrical System

Xedos 6, 2.0i
Alternator output: 90/3000 Amps/eng/rpm
Regulated output: 13.8-14.4 Volts
Battery: 12/58 V/Ah
Starter motor make/type: Bo/0 001 108 072

Other data

Xedos 6, 2.0i
Clutch operation type: Cable
Clutch adjustment: 5-14/Pedal mm
Cambelt failure: Damage
Cambelt Part Number: n / a
Tyre pressures: 2.0(28)f 2.0(28)r bar (psi)