Nederlands

Conversie Mazda 323F Motor code : Z5 naar BP05

Conversies  Mazda 323F BA 1.5 met één Mazda 323F BG 1.8 16v DOHC motor

Materialen : 

- BG 1.6 SOHC aandrijfas incl. fusee en kogel  rechts.

- BG 1.8 DOHC 16v BP motor.
- BG 1.8 DOHC stappenmotor stekker met ± 6cm draad.
- BA 1.8 DOHC Luchtmassa meter.

- BA 1.8 DOHC motorkabelboom, incl. injectie rail kabelboom.

- BA 1.8 DOHC ECU.

- BA 1.8 DOHC Lambda sensor
- BA 1.5 DOHC versnellingsbak.

- BA.1.5 DOHC aandrijfas rechts.

- Catalysator vervanger, of pasmakend

Pasmaken → BG motor → BA 1.8 DOHC kabelboom

| - Stappenmotor stekker → BG | BA |
| --- | --- |
| Zwart | Blauw |
| Groen | Groen |
| Rood | Zwart |

- EGR	→ Logische kleuren komen overeen in zowel de BG als de BA kabelboom

- Motorsteun rechts → Complete motorsteun van de BA overzetten op de BG motor.  Alleen het gietijzeren deel met draadeinden blijft van de BG.

- Koelslang T.B.V. radiateur →  Hierbij de originele gebruiken. Inkorten aan de motorkant rechts van de radiateur.

Made by
Adri Hoogenbosch
H-P-C

English

Conversion Mazda 323F enginecode : Z5 to BP05

Conversion  Mazda 323F BA 1.5 with an Mazda 323F BG 1.8 16v DOHC engine

Materials : 

- BG 1.6 SOHC transaxel including wheelhub rightside.

- BG 1.8 DOHC 16v BP05 engine.
- BG 1.8 DOHC steppermotor plug with  approx 2”  threat.
- BA 1.8 DOHC airflowmeter.

- BA 1.8 DOHC engine wiringloom including injectionrail loom.

- BA 1.8 DOHC ECU.

- BA 1.8 DOHC exhaustsensor
- BA 1.5 DOHC transmission.

- Catalysator replacement or make the original fit.

Make it fit → BG engine → BA 1.8 DOHC wiringloom

| - Steppermotor plug → BG | BA |
| --- | --- |
| black | Blue |
| green | green |
| red | Black |

- EGR	→ Logic colours will fit the BG wiringloom with the BA wiringloom

- Engine bracket rightside → Use the complete engine bracket from the BA on the BG engine. Only the castiron part with the screwthreads stays on the BG engine. See picture at the rightside of this page.

- Cooling hose for the radiator →  Use the original hose, you only have to makei t a bit shorter at the engine side.

* Marker :  Steppermotor, fitting on the trotthle body, a electric motor designed to provide displacement or speed variation in successive steps

Translated from Dutch to English
Made by
Adri Hoogenbosch
H-P-C